VH-155 power bank

- specification book

1.Scope of application

Suitable for most USB input electronic devices.

2. Product features

2.1 quick charging, low power consumption and long power consumption.

2.2 safety and reliability: multiple protection (charge control, charge protection, discharge protection, overload protection and short circuit protection) of intelligent control circuit inside the product can solve your worries about using the product.

2.3 high-efficiency power retention: static power consumption of less than 250uA (microamps), left for one month after full charge, still has more than 90% power.

2.4 strong compatibility: it can charge various brands of mobile phones, MP3, MP4 and other digital products, only need to be equipped with matching adapter.

2.5 convenient and fashionable, charging anytime and anywhere, no trouble of power failure.

3. The basic parameters

| specification | NO. | 155 |
| --- | --- | --- |
|  | Size(mm) | 12.5*63*90mm |
|  | Weight（g） |  |
|  | Batteries specifications | 955565*1 |
|  | capacity（mAh） | 5000mA |
|  | Output | Output: 5V±0.3V(2000mA) |
|  | color | customzied |
|  | Power indicator light | White |
| Charge characteristic | Charging mode | 线性， 恒流/恒压（CC/CV） |
|  | Charging input voltage（V） | 5.0V±5% |
|  | Charging current（mA） | 2000mA(max) |
|  | Lithium battery charge limit voltage（V） | 4.20V |
|  | Lithium battery full voltage（V） | 4.15V～4.25V |
|  | Charging cut-off current | ≤100mA |
| Discharge characteristic | The output voltage（V） | The operation mode; 5.0V±0.3/1000mA(max) (Constant current mode). Standby mode; 5.0V±0.3V. Core protection mode (over current, over discharge, short circuit, etc.); 0V. |
|  | power consumption（uA ）(Sleep patterns) | ＜250uA |
|  | Discharge efficiency | average efficiency≥70%(25度) |
|  | Protection function | Overrelease protection, after short circuit protection (press the switch, the display light will be on, continue to output) |
|  | Output interface |  |
|  | Input and output | Micro USB input interface is used to charge the power, directly plug in the load to automatically identify the output.Press the switch and charge other devices with the USB output port |
| Power protection characteristic | Overcharge protection voltage（V） | 4.28±0.025(second protection) |
|  | Current limiting protects current（A） | 2～4.5 |
|  | Over discharge protection voltage（V） | 2.90±0.05 |
| Cycle life |  | Number of charge and discharge cycles≥300次 |

4、Environmental conditions

4.2.1 Operating temp：		   Charge temp	0～+45℃

Discharge temp	-20～+65℃

4.2.2 Suitable temp：					-5~+45℃

5、transport

The power supply should be packaged into boxes under the semi-charged state (50 ~ 70% charging state) for transportation. During the transportation process, it should be protected from severe vibration, shock or extrusion, sun and rain. It should be suitable for transportation by common means of transportation such as cars, trains, ships and planes.

6. Storage method

Long term storage (more than 3 months), the battery should be kept at 50% of the rated capacity (must be charged once in 3 months), stored in a dry and cool place, the temperature range between -10℃ ~ 30℃, and avoid contact with corrosive substances, away from fire and heat sources.

7. Packaging

The packaging method of the battery shall be in accordance with the drawing or packaging requirements provided by the customer, and the mode of transportation shall be in accordance with the relevant provisions of the purchase and sales contract between the parties.

8. Safety procedures and instructions

Please read the following instructions before use. Improper use will cause heating, fire, breakage, damage and battery power degradation.

1. Please do not store the product in a high temperature environment above 45℃, and do not expose it to the sun. Please do not put the product into fire or put it in other overheating environment.

2. Do not disassemble this product or try to carry out any kind of maintenance or modification on it. Do not try to replace any part of this product.

3. Do not drive nails into the battery or hit it with a hammer.

4. Do not soak the battery in water or soak the power supply when stored.

5. This product will have appropriate heating phenomenon in the charging process, and the temperature is below 45℃, which will affect the product performance. Please rest assured to use it.

6. All products have passed strict quality assurance inspection before leaving the factory.If you find this product overheating, odor, deformation,wear, damage or other abnormal conditions, please immediately stop using, and contact the manufacturer or product agent.

7. If the purchaser purchases the product for use by minors, the purchaser shall provide detailed instructions and warning information to the minors before use.If buyer fail to fulfil demonstrative obligation, manufacturers not minors the consequences of improper use or misuse any liability (the mobile power supply is not suitable for physiological, perception and mental ability to people with low (including children) to use, does not suggest a lack of experience and knowledge, unless there is a special person responsible for the safety of their life of supervision or guidance they use the equipment.Watch your children to make sure they don't use the portable power supply as a toy.)

8. The battery should be stored in a cold and dry environment without charging. The battery must be discharged before storage or transportation.

9, if the battery is used in the limit conditions, may reduce the battery life, such as: limit temperature, deep cycle, extreme over-charge and over-discharge.